1.Transistor circuit

-guess and sketch the waveform of VA and VB

Reason:

Query the datasheet and we could find that the gate threshold voltage is about 2.1V.

VA is always almost equal to the input voltage.

When the input voltage < 2.1V, the NMOS is on, so VB=VDD=5V.

And when the input voltage > 2.1V, the NMOS is off, so VB=0V.

- Build the circuit in Multisim, use ‘DC sweep’ or ‘transient’ simulation to validate your guess.

- Find the output waveform of VB, and calculate the voltage gain.

Enlarge the wave of VB, we could find that when VB≈2.5V, the input voltage≈2.222743V.

So let the input voltage=2.222743V, and print the waveform of VB.

Amplitude of the VB’s wave is 2.523V-2.501V=0.022V

And the Amplitude of the peak voltage is 1mV=0.001V

So the voltage gain is (2.523-2.501)/0.001=22

The voltage gain=22.

2.Operational amplifier

Circuit #1

Vout=-1V

Circuit #2

Vout=-7V

3.Analog-to-digital converter (ADC)

the simplifications of digital circuit:

The circuit is as followed.

Change the value of Vin means the change of Vin.

If B1/B0=1, then the corresponding light will light up.

If B1/B0=0, then the corresponding light will extinguish.

4.MCU development

from machine import TouchPad,Pin#import  TouchPad,Pin
import time#import time for sleep

red=Pin(27,Pin.OUT)#let the red led get source from D27
yellow=Pin(26,Pin.OUT)#let the yellow led get source from D26
green=Pin(25,Pin.OUT)#let the green led get source from D25
touch=TouchPad(Pin(13))#let the TouchPad get data from D13
while 1:#follow the circuit
  while touch.read()>1000:#the hand do not touch the pin
    for i in range(5):#the red led will be on for 5 seconds
      if touch.read()<1000:#the hand is touching the pin
        break#reset the whole system
      red.value(1)#let the red led be on
      sleep(0.5)#it will be on for 0.5 second
      red.value(0)#let the red led be off
      sleep(0.5)#it will be off for 0.5 second
    if touch.read()<1000:#the hand is touching the pin
      break
    for i in range(2):#the yellow led will be on for 2 seconds
      if touch.read()<1000:#the hand is touching the pin
        break#reset the whole system
      yellow.value(1)#let the yellow led be on
      sleep(0.5)#it will be on for 0.5 second
      yellow.value(0)#let the yellow led be off
      sleep(0.5)#it will be off for 0.5 second
    if touch.read()<1000:#the hand is touching the pin
        break#reset the whole system
    for i in range(4):#the green led will be on for 4 seconds
      if touch.read()<1000:#the hand is touching the pin
        break#reset the whole system
      green.value(1)#let the green led be on
      sleep(0.5)#it will be on for 0.5 second
      green.value(0)#let the green led be off
      sleep(0.5)#it will be off for 0.5 second